BusLogic QuickTech Guides

SCSI Hard Drives

BusLogic SCSI Host Adapters can support up to 7 SCSI Hard Drives per Controller (ID’s 0-6), provided your operating system supports that quantity of drives. If you have the BIOS enables, ID 0 & 1 are reserved for Hard Drives and will allow you to boot directly off of the SCSI Hard Drive, so depending on how many drives you want in your system, do the following:

| Number of Hard Drives | Action |
| --- | --- |
| 1 | Place Hard Drive on ID 0, will be your boot device, Load BTDOSM.SYS or Operating System Driver for Better Performance |
| 2 | Place Hard Drive 1 on ID 0, Place Hard Drive 2 on ID 1, Load BTDOSM.SYS or Operating System Driver for Better Performance |
| >2 | Place Hard Drive 1 on ID 0, Place Hard Drive 2 on ID 1, Place other Drives on ID 2-6. Under DOS --------------- In your CONFIG.SYS, add line: DEVICE=BTDOSM.SYS /D DEVICE=BTMDISK.SYS Use BTFDISK to FDISK and FORMAT to prep the drives. Under other OS’s ----------------------- Load Device Driver, will provide multiple DASD Support |

You may need to do a low-level format on your drive to properly prep the drive. On non-C product boards, go into debug and type G=DC00:6 (Provided the base address is DC00), then perform a low level format.