c-------------------------------------------------------------------------------

c

c                                   CurveFit

c

c         A general curve fitting routine using PGPLOT graphics.  It works on

c      VAX/uVAX systems, possibly any system running PGPLOT.

c

c         CurveFit will plot up to 30 data sets (a maximum of 2500 points for

c      each data set, a maximum of 24000 points total for one graph), saved

c      as (xp,yp) in ascending {xp} order.  Different data sets in one file

c      must be separated by one non-numeric line.  CurveFit can also modify the

c      data set, by flipping the x and y axes and/or taking logarithms of

c      either or both of them.  Labels and title can be added to the plots.

c      In addition, CurveFit allows one to fit various types of curves to the

c      data sets.  These are:

c

c           (a)  straight line (connect the dots)

c           (b)  cubic spline

c           (c)  smoothing (Bezier) polynomial

c           (d)  low-pass Fourier transform filter

c           (e)  a best-fit polynomial of up to 10 terms

c

c         CurveFit is menu driven and allows one to display all intermediary

c      results.  The x,y plotting ranges are user determined, as is the

c      fitting range (over the x coordinate).  Each type of fit can only be

c      used once with each data set, though different fits may be used with

c      the same data set (however, one could load the data set in again and do

c      the same fit over a different range).  Different data sets can have

c      different symbols and line styles/widths associated with them, so they

c      can be differentiated.  The manner in which the line styles/widths is

c      determined can be changed if the user so desires, as can the symbols

c      used to plot the data points.  Plots can be made on any device that is

c      supported by PGPLOT.

c

c         The programme is fairly well documented.  A subsantial amount of

c      virtual memory is required (of the order of 7 Mb), but this can be

c      decreased by lowering the number of data points that may be plotted.

c      The following are pseudo-global variables that must be changed in each

c      subroutine in order for the programme to work.

c

c           MAXPOINTS    the maximum number of points allowed per graph

c           MAXPOINTSET  the maximum number of data points per data set

c           MAXDATASET   the maximum number of data sets per graph

c           NCOEFF       the maximum number of coefficients for polynomial

c                            fitting

c           S2           gives the extra number of points calculated by the

c                            curve fitting routines to make the curves look

c                            smooth, given as 4*MAXPOINTS or 8*MAXPOINTS

c

c         The Bezier polynomial fitting is taken from Borland's TURBO Graphix

c      Toolbox (translated into FORTRAN), while the cubic spline routines

c      are partially taken from there, and partially my own algorithm.  The

c      polynomial fitting and the fast Fourier transforms are taken from

c      the book Numerical Recipes by W.H. Press et al.  Except as noted below,

c      the routines all seem to do what they are supposed to.  Note that the

c      polynomial curve fitting gives the coefficients and their standard

c      deviation as well.  Take these values with a grain of salt!

c

c         Currently, everything seems to work fairly well, except for some

c      numerical difficulties with the fast Fourier transform that I am

c      unable to fathom.  The problem only becomes noticeable when the FFT

c      smoothing takes place over more than about 100 data points, when

c      anomalous step functions appear at intervals of powers of 2.  The

c      step functions get larger as more data points are being smoothed over.

c      I would appreciate a solution if anyone comes up with one.

c

c         One other minor difficulty discovered occurs when one is taking

c      logs of more than one data set.  It is possible that the endpoints

c      used for the plotting will not cover the full range of all of the

c      data sets plotted.  A fix would require several more variables, plus

c      thinking, of which I do not have time at the present.

c

c         Distribute and use this programme as you see fit.  If anyone comes

c      up with major changes or improvements, please send me a copy of the

c      source code.  If anyone wishes a current copy, send me a note, and

c      the current source code will be sent out to them.  I can be reached

c      at

c

c            BITNET:    gill@qucdnast

c            INTERNET:  gill@qucdnast.queensu.ca

c

c

c  October 27, 1987 -     Fix of subroutine Spline1.  It was able to crash

c                     with a particular spacing of {xp}.  It was also found

c                     to be incorrect.

c                         Fix to all calculation subroutines, so that the

c                     fit will be made over only the points INSIDE the

c                     given fit range, inclusively, i.e. no extrapolation.

c  October 28, 1987 -     Fix of Subroutine Spline2.  It gave discontinuous

c                     spikes at the endpoints.

c                         Add double precision calculations to Spline1 and

c                     Spline2 to allow accurate subtractions/additions.

c  November 6, 1987 -     Fix of calculation procedures that give the range

c                     of the plot.  These are now given to 2 digits accuracy.

c                         Add the option to plot or not plot the individual

c                     data points.

c  November 13, 1987 -    Fix the x,y plotting limits when new data sets are

c                     loaded.

c                         Each {xp,yp} pair must be on the same line.  Other

c                     information is now allowed on each line after {xp,yp}.

c  March 8, 1988 -        Add READDATA subroutine, that allows one to:

c                     a) skip lines at the beginning before data input,

c                     b) have up to 25 columns of data that can be accessed,

c                     c) read multiple data sets from the same file.

c  April 21, 1988 -       Fix of number of points associated with the data

c                     set number.

c                         The default video and hardcopy devices are now

c                     /VT and /IM, respectively.

c  May 30, 1988 -         Add the option of user inputed line styles/widths.

c  June 2, 1988 -         Correct the problem of bad standard deviations on the

c                     polynomial coefficients.

c                         Add REAL*8 and /G_FLOATING to all of the polynomial

c                     calculations.

c  June 13, 1988 -        Add some idiot-proofing (read `Error checking') of

c                     the curve-fitting routines.

c  July 27, 1988 -        Correct the assignments of the fitting range

c                     variables.

c  August 29, 1988 -      Add error-trapping of non-character data input.

c                         Add scaling (in x and/or y) of any data set.

c                         Add ability to toggle all line drawing and fits for

c                     any data set at any time.

c                         Add user chosen line style/line width combinations.

c                         Add user chosen data point plot symbols.  The size of

c                     these is regulated by the total number of points (more

c                     points gives smaller symbols).

c  Christmas, 1988 -      Rearrange the storage method for the data.  Now

c                     there is a maximum of 24000 points, or 30 data sets of

c                     2500 points apiece.  This should allow for more

c                     flexibility for the user.

c  June 21, 1989   -      Change the entrance into the routines to allow

c                     multiple changes to data sets without going to the

c                     main menu screen in between each change.

c                         Fix of the log routines.  Check that your machine

c                     will assign 0 to 10^-100.

c  June 22, 1989   -      Fix of number of points plotted for various fits.

c                     Now each fit may have a different number of data points.

c  July 4, 1989    -      Fix of spline calculation routines.  Splines of more

c                     than one dataset were not saved correctly or plotted.

c  July 6, 1989    -      The default line width, line style will now always

c                     be 1 and 1.

c  July 11, 1989   -      Updated the help file subroutine.

c  July 14, 1989   -      Correct the plot symbol choosing routine.

c                         More idiot-proofing.

c                         Add an UpCase function (works with ASCII only).

c  August 29, 1989 -      Change the order of commands so that the main menu

c                     will not automatically be drawn between commands.  To

c                     redraw menu, type in `R' or `M'.

c  August 31, 1989 -      Fix READDATA, so that any data read in will be from

c                     one line only.  Before, multiple lines were permitted.

c                     Also, an error message is given if the asked for column

c                     does not exist.  The column can then be chosen again.

c

c         Last revision - August 31, 1989 - Arnold Gill

c         Last revision sent to Tim Pearson (tjp@citphobo) - August 31, 1989

c

c-------------------------------------------------------------------------------