Black Horse Controller

by

Jarkko Vuori, OH2LNS

email: Jarkko.Vuori@hut.fi

These programs are beta versions of the software written for the Motorola

M68HC11 based microcontroller board called as 'Black Horse Controller'.

Black Horse Controller is simply MC68HC11A1 MCU with Texas TMS29F256 32KB

FLASH EPROM at address 8000H - FFFFH and 32KB RAM at address 0000H - 7FFFH.

SCI interface is connected via MAX232 level shifter chip to the host computer

so that the host computer can reset the MCU to the special boostrap mode

(host computer can toggle with DTR line MODA, MODB and RS pins). No other

peripherals or connections are needed in order to operate these software.

There are programs for downloadin Motorola S-record to the MCU and alter

its CONFIG register.

There is also available a PD multitasking operating system, called Multinel,

which is ported to this Black Horse Controller.

These programs are intended for private use only. You may redistribute far

and wide but charge no money, not even handling fees.

Commercial use of this code is absolutely prohibited without permission of

the authors. This code may not be used as the basis of any program that will

be sold, bartered, or in any way traded for gain without the permission of

the authors.

There is no warranty. What you see is what you get.

EQUIPMENT REQUIREMENTS

This software is written for Motorola M68HC11A1 based Black Horse Controller.

This code is written with Microsoft MSC 5.1 and Microsoft Make utility and

IAR Cross Assembler for 68HC11 processors.

OTHER INFO