From rwstephens@ptbo.igs.net Wed Apr 24 19:55:58 1996

mtc01rws.jpg

This is me standing in my test room with my newly built medium sized

Tesla coil (November'95)shown here with 8 inch by 30 inch toroid.

Secondary is #18 AWG, PVC insulated wire, closewound to a length of

4 feet on a 15 inch diameter, 1/2 inch wall  PVC form. Unit operates

at about 100KHz.  Rotary break at 400 PPS.  Present maximum power

capability 4.6 kVA (20 amps@230 volts) was used in this photo group.

Has produced up to 9 foot streamers at this power level. This unit is

a production prototype demonstrator from my line of custom built Tesla

coils available for sale.

mtc02rws.jpg

Medium TC in unusual neat shot of single horizontal streamers under full room

lighting.

mtc03rws.jpg

Medium TC with larger 10 inch by 35 inch topload being tested.  7.5 foot

streamers to overhead  grounded strike wires.

mtc04rws.jpg

Medium TC. Six foot streamers to right strike curtain.

mtc05rws.jpg

Medium TC.  8.2 foot streamer to strike curtain in upper right.

mtc07rws.jpg

First outdoor test of my medium T.C. at increased input power of 6.9

kVA(30A @ 230V), and with new, larger 12" x  48" toroid.  A measured

twelve foot point-to-point streamer darts off  frame to the left to

connect with the tin roof of my shop.

mtc08rws.jpg

First outdoor test of my medium T.C. at increased input power of 6.9

kVA (30A @ 230V), and with new, larger 12"x 48" toroid. Many strikes

to protective ground wire and one to the primary strike ring.

mtc09rws.jpg

First outdoor test of my medium T.C. at increased power of 6.9

kVA (30A @ 230V), and with new, larger 12"x 48" toroid.  Destructive

testing of a wooden ladder and 4 foot fluorescent lamp in progress.

mtc11rws.jpg

First outdoor test of my medium T.C. at increased power of 6.9

kVA (30A @ 230V), and with new, larger 12"x 48" toroid.  This photo

proves that you don't have to "connect" to a fluorescent lamp to

illuminate it.

mtc12rws.jpg

This is my hand.  This is my hand on my medium T.C.  Any questions?