H P   4 8 S X   I n t e r n a l s

Derek S. Nickel

23 February 1991

================================================================

Forward

================================================================

This documnent describes some of the internals of the HP 48SX

calculator.  Its intended audience is people with a good under-

standing of the basic HP 48SX who want to explore the internals

of the HP 48SX.

The information contained here is from a variety of

sources:  HP-71B Internal Design Specifications, ROM analysis

and other posted information.

Nothing is perfect.  Double check these notes if you plan to use

the information that they contain.

Send comments and questions to:

Derek S. Nickel

15 Maple Lane

Walnut Creek, CA 94595-1718

================================================================

Usage Notes

================================================================

In the examples, when ASC\-> is used, the last four characters

are the checksum and are not part of the object.  Note that

when atomic objects are viewed as a string (via \->ASC or the

Memory Scanner) they appear backwards.  This is because the

string representation is viewed least-significant-nibble to

most-significant nibble in left to right order, whereas binary

data is viewed in right to left order.

Most of the examples list the internal representation as:

12345 6789 ABC DEF0

This corresponds to a memory dump ('48 style) of:

xxxxx: 543219876CBA0FED

or a HEX dump (VAX/VMS style) of:

DE F0 AB C6 78 91 23 45 E#.x....         xxxxx

================================================================

Prologs

================================================================

In the HP 48SX, there are many different data types.  Some are

common and easy to use: Real Number, Complex Number, Array,

String, Binary Integer, List, Directory, Algebraic, Unit,

Tagged, Graphic, Backup, Program, Global Name and Local Name.

Some are common but hard to create:  Library, Library Data and

XLIB Name.  And some, at the user level are nonexistant:

System Binary, Long Real, Long Complex, Character, Linked Array

and Code.

One thing that all of these data types have in common is a five

nibbles header. This header is called a prolog and it identifies

the data type of the data that follows the header.  The prolog

is the address of a machine language that 'handles' the data.

In most cases, this just involes putting a pointer to the object

on to the data stack.  Thus, when you evaluate an object that is

on the data stack, it is removed from the data stack and then it

puts itself back onto the data stack.  Some objects, though,

have a different behaviour.  Like programs; they evalute their

contents (among other things).

Each prolog routine starts with a five nibble 'prolog signature'

(028FC).  This is not an address, but an unusual seqence of

machine language instructions.

Voyager> ua 2933

02933: ; *** Real Number ***

02933: CF         D=D-1  A

02935: 820        HST&=  #F

02938: 465        GOC    0298F   ; MC XFER: GC, push A.A and

; continue RPL

0293B: 1C4        D1=D1- 5       ; push on to data stack

0293E: 141        DAT1=A A       ; ...

02941: 142        A=DAT0 A       ; continue RPL

02944: 164        D0=D0+ 5

02947: 808C       PC=(A)

Voyager>

================================================================

Prologs (by prolog address)

================================================================

Prolog	Object Name	Type #

------	--------------	------

02911	System Binary	  20

02933	Real Number	   0

02955	Long Real	  21

02977	Complex Number	   1

0299D	Long Complex	  22

029BF	Character	  24

029E8	Array		 3,4

02A0A	Linked Array	  23

02A2C	String		   2

02A4E	Binary Integer	  10

02A74	List		   5

02A96	Directory	  15

02AB8	Algebraic	   9

02ADA	Unit		  13

02AFC	Tagged		  12

02B1E	Graphic		  11

02B40	Library		  16

02B62	Backup		  17

02B88	Library Data	  26

02BAA	iType <BFh> object

02BCC	iType <CFh> object

02BEE	iType <DFh> object

02C10	iType <EFh> object

02D9D	Program		   8

02DCC	Code		  25

02E48	Global Name	   6

02E6D	Local Name	   7

02E92	XLIB Name	  14

================================================================

Prologs (by type number)

================================================================

Type #	Prolog	Object Name

------	------	--------------

0	02933	Real Number

1	02977	Complex Number

2	02A2C	String

3	029E8	Real Array

4	029E8	Complex Array

5	02A74	List

6	02E48	Global Name

7	02E6D	Local Name

8	02D9D	Program

9	02AB8	Algebraic

10	02A4E	Binary Integer

11	02B1E	Graphic

12	02AFC	Tagged

13	02ADA	Unit

14	02E92	XLIB Name

15	02A96	Directory

16	02B40	Library

17	02B62	Backup

18	none	Command

19	none	Function

20	02911	System Binary

21	02955	Long Real

22	0299D	Long Complex

23	02A0A	Linked Array

24	029BF	Character

25	02DCC	Code

26	02B88	Library Data

27	other	Unknown prolog

Commands and Functions can be expressed as either their address

or as XLIB Names.  For example, DUP is normallay represented by

the address 1FB87, but it can also be represented by XLIB 2 269

(02E9E 002 10D).

================================================================

Internal Object Type Numbers

================================================================

The following table lists the internal object types (iTypes) and

the objects that they match.  These iTypes are used in the

internal RPL operations to do argument type checking.  Note that

if the first nibble is an F, then a second nibble follows.

Also, if a operation requires more that one argument, then each

argument will be represented as a digit in the Systen Binary

that is used for argument type checking.  For example, the

System Binary number <C15h> would mean that stack level one

must contain a List, stack level two must contain a Real Number

and stack level three must contain a Graphic.

One nibble iTypes:

iType	Objects

------	--------------------------------------------------------

<0h>	Any

<1h>	Real Number

<2h>	Complex Number

<3h>	String

<4h>	Array

<5h>	List

<6h>	Global Name

<7h>	Local Name

<8h>	Program

<9h>	Algebraic

<Ah>	Symbolic (Algebraic or Global Name)

<Bh>	Binary Integer

<Ch>	Graphic

<Dh>	Tagged

<Eh>	Unit

Two nibble iTypes (starts with F):

iType	Objects

------	--------------------------------------------------------

<Fh>	XLIB Name

<1Fh>	System Binary

<2Fh>	Directory

<3Fh>	Long Real

<4Fh>	Long Complex

<5Fh>	Linked Array

<6Fh>	Character

<7Fh>	Code

<8Fh>	Library

<9Fh>	Backup

<AFh>	Library Data

<BFh>	prolog <02BAAh>

<CFh>	prolog <02BCCh>

<DFh>	prolog <02BEEh>

<EFh>	prolog <02C10h>

<FFh>	?? <191BEh>

================================================================

Atomic Data Types

================================================================

Atomic data types are listed after each field description, in

parenthesis.  Note that some atomic data types are more atomic

than others.

**** ASCIC ****

ASCII counted character strings are represented by length and

data fields.  ASCII counted character strings are the basis for

Global Name, Local Name and Tagged data types, as well as

components in other objects.

N          2  1     0

+-------------+--------+

| String Data | Length |

+-------------+--------+

Length        2

where N = Length/2 + 1.

**** ASCII ****

ASCII character strings are represented as a sequence of bytes

(ASCII characters).  The length of the ASCII character string is

specified elsewhere.  ASCII character strings are called ASCII

in the field descriptions.  ASCII character strings are the

basis for String data types.

N          0

+-------------+

| String Data |

+-------------+

Length

**** ASCIX ****

Extended ASCII counted character strings are represented by

length and data fields.  If the length is zero, only a single

length field is present.  If the length is non-zero, then there

are two identical length fields, one before and one after the

string data.  Extended ASCII counted character strings are the

used in Directory, Library and Backup data types.

N   N-1 N-2         2  1     0

+--------+-------------+--------+

| Length | String Data | Length |

+--------+-------------+--------+

2        Length        2

where N = Length/2 + 2.

Atomic Data Types (cont)

Or, if length is zero:

1 0

+----+

| 00 |

+----+

2

**** BCDx ****

BCD numbers (binary coded decimal) are represented by a mantissa,

sign and exponent fields.  These are two kinds of BCD numbers in

the HP48SX, 12-form and 15-form BCD numbers, called BCD12 and

BCD15 in the field descriptions.

Both kinds have several things in common:

*	A mantissa.  Each digit of the number is represented by

one nibble in the mantissa.  12-form BCD numbers have a

12 digit mantissa and 15-form BCD numbers have a 15

digit mantissa.  There is an implied decimal point after

the Most Significant Digit (msd) with respect to the

exponent.

*	A sign nibble.  Positive numbers are represented by a

0 and negative numbers are represented by a 9.

*	An exponent.  The exponent is in 10's compement.

12-form BCD numbers have a 3 nibble exponent and 15-form

BCD numbers have a 5 digit exponent.

12-form BCD numbers are the basis for Real Number, Complex

Number, Real Array and Complex Array data types.

15-form BCD numbers are the basis for Long Real and Long

Complex data types.

BCD12:

15 14                     3  2   0

+--+------------------------+------+

|S | msd..  Mantissa  ..lsd | Exp  |

+--+------------------------+------+

1             12              3

BCD15:

21 20                           5  4       0

+--+------------------------------+----------+

|S | msd...    Mantissa    ...lsd |   Exp    |

+--+------------------------------+----------+

Atomic Data Types (cont)

**** BINx ****

Binary integers are represented by a data field.  Binary integers

are called BINx in the field descriptions (where x = the number

of nibbles that the number occupies).  Binary integers are the

basis for Binary Integers and System Binary data types.

N          0

+-------------+

| Binary Data |

+-------------+

x

where N = x - 1.

**** CHAR ****

ASCII characters are represented by a one byte (two nibble) data

field.  ASCII characters are called CHAR in the field

descriptions.  ASCII characters are the basis for the Character

data type.

1        0

+-----------+

| Character |

+-----------+

2

**** CODE ****

Machine code is represented as a sequence of nibbles that is the

machine code.  Machine code is called CODE in the field

descriptions.  Machine code is the basis for the Code data type.

Examples in this document use the HP mneumonics.

================================================================

System Binary (02911)

================================================================

<prolog><data>

<prolog> = 02911 (BIN5)

<data> = five nibble binary integer (BIN5)

There are several tables of System Binary numbers:

addr	  HEX    description

------	-------  ------------------------------------------------

03F8B	<2933h>  Real Number prolog

03F95	<2977h>  Complex Number prolog

03F9F	<2A74h>  List prolog

03FA9	<2911h>  Global Name prolog

03FB3	<2D9Dh>  Program prolog

03FBD	<2AB8h>  Algebraic prolog

03FC7	<2A96h>  Directory prolog

03FD1	<2E6Dh>  Local Name prolog

03FDB	<2955h>  Long Real prolog

03FE5	<2ADAh>  Unit prolog

addr	 HEX	 DEC

------	------	------

03FEF	<0h>	<0d>

03FF9	<1h>	<1d>

04003	<2h>	<2d>

0400D	<3h>	<3d>

04017	<4h>	<4d>

04021	<5h>	<5d>

0402B	<6h>	<6d>

04035	<7h>	<7d>

0403F	<8h>	<8d>

04049	<9h>	<9d>

04053	<Ah>	<10d>

0405D	<Bh>	<11d>

04067	<Ch>	<12d>

04071	<Dh>	<13d>

04073	<Eh>	<14d>

04085	<Fh>	<15d>

0408F	<10h>	<16d>

04099	<11h>	<17d>

040A3	<12h>	<18d>

040AD	<13h>	<19d>

040B7	<14h>	<20d>

040C1	<15h>	<21d>

040CB	<16h>	<22d>

040D5	<17h>	<23d>

040DF	<18h>	<24d>

040E9	<19h>	<25d>

040F3	<1Ah>	<26d>

040FD	<1Bh>	<27d>

04107	<1Ch>	<28d>

04111	<1Dh>	<29d>

0411B	<1Eh>	<30d>

04125	<1Fh>	<31d>

System Binary (cont)

0412F	<20h>	<32d>

04139	<21h>	<33d>

04143	<22h>	<34d>

0414D	<23h>	<35d>

04157	<24h>	<36d>

04161	<25h>	<37d>

0416B	<26h>	<38d>

04175	<27h>	<39d>

0417F	<28h>	<40d>

04189	<29h>	<41d>

04193	<2Ah>	<42d>

0419D	<2Bh>	<43d>

There are several routines to convert between System Binary and

other data types.  No error checking is performed by these

routines.

addr	 name	description

------	------	------------------------------------------------

18CEA	R\->SB	Real Number to System Binary

18DBF	SB\->R	System Binary to Real Number

Example:

Create a System Binary with a decimal value of 48.

02911 00030

"1192003000C53E" ASC\-> returns <30h>

- or -

48 #18CEAh SYSEVAL returns <30h>

================================================================

Real Number (02933)

================================================================

<prolog><data>

<prolog> = 02933 (BIN5)

<data> = 12-form BCD number (BCD12)

There are several tables of Real Number and Long Real numbers:

addr	 value

------	----------------

2A2B4	0

2A2C9	1

2A2DF	2

2A2F3	3

2A308	4

2A3D1	5

2A332	6

2A347	7

2A35C	8

2A371	9

2A386	-1

2A39B	-2

2A3B0	-3

2A3C5	-4

2A3DA	-5

2A3EF	-6

2A404	-7

2A419	-8

2A42E	-9

2A443	3.14159265359

2A458	3.14159265358979 (Long Real)

2A472	9.99999999999E499

2A487	-9.99999999999E499

2A49C	1.E-499

2A4B1	-1.E-499

2A4C6	0 (Long Real)

2A4E0	1 (Long Real)

2A4FA	2 (Long Real)

2A514	3 (Long Real)

2A52E	4 (Long Real)

2A548	5 (Long Real)

2A562	.1 (Long Real)

2A57C	.5 (Long Real)

2A596	10 (Long Real)

There are several routines to convert between Real Number and

other data types.  No error checking is performed by these

routines.

addr	 name	description

------	------	------------------------------------------------

2A5B0	LR->R	Long Real to Real Number

2A5C1	R->LR	Real Number to Long Real

Real Number (cont)

Example:

Create a Real Number with the value of the golden mean:

02933 0161863398875000

"3392000057889330816106AB0" ASC\-> returns

1.61863398875

================================================================

Long Real (02955)

================================================================

<prolog><data>

<prolog> = 02955 (BIN5)

<data> = 15-form BCD number (BCD15)

================================================================

Complex Number (02977)

================================================================

<prolog><real-part><imaginary-part>

<prolog> = 02977 (BIN5)

<real-part> = 12-form BCD number (BCD12)

<imaginary-part> = 12-form BCD number (BCD12)

================================================================

Long Complex (0299D)

================================================================

<prolog><real-part><imaginary-part>

<prolog> = 0299D (BIN5)

<real-part> = 15-form BCD number (BCD15)

<imaginary-part> = 15-form BCD number (BCD15)

================================================================

Character (029BF)

================================================================

<prolog><data>

<prolog> = 029BF (BIN5)

<data> = ASCII coded character (CHAR)

================================================================

Array (029E8)

================================================================

<prolog><size><item-prolog><#dims><dim-1>...<dim-n><item-1>...

<item-m>

<prolog> = 029EB (BIN5)

<size> = number of nibbles remaining (BIN5)

<item-prolog> = prolog for array elements (BIB5)

<#dims> = number of array dimensions (BIN5)

<dim-i> = i-th dimension (BIN5)

<item-j> = j-th item (ANY)

n = <#dims>

m = <dim-1> * <dim-2> * ... * <dim-n>

The items in the array are stored in Row Major order (all of the

first rows elements are group together, followed by the second

row, and so forth.)

In normal calculator operations:

*	Only one and two dimensional arrays are supported.

*	Only Real Numbers and Complex Numbers are supported as

array elements (items).

Some assumtions are made about various fields with normal

operations:

*	If <item-prolog> is not Real Number, then its assumed to

be Complex Number.

*	If #Dims is not 1, its assumed to be 2.

This applies to the Matrix Editor, GET and so forth.

But the '48 uses other types of arrays internally.  For example,

message tables are "Array of String".

================================================================

Linked Array (02A0A)

================================================================

<prolog><size><???>

<prolog> = 02A0A (BIN5)

<size> = number of nibbles remaining (BIN5)

<???> = undetermined...

================================================================

String (02A2C)

================================================================

<prolog><size><data>

<prolog> = 02A2C (BIN5)

<size> = number of nibbles remaining (BIN5)

<data> = (<size>-5)/2 byte character string (ASCII)

================================================================

Binary Integer (02A4E)

================================================================

<prolog><size><data>

<prolog> = 02A4E (BIN5)

<size> = number of nibbles remaining (BIN5)

<data> = binary integer (BINx, where x = <size> - 5)

Most Binary Integers have a data size of 16 nibbles or 64 bits

(<size> = 21).  But some have a smaller data size.  For example,

the BYTES command returns a Binary Integer (the checksum) with a

data size of 4 nibbles or 16 bits (<size> = 9), whereas the Hash

Table in a Library looks like a very large Binary Integer.

================================================================

List (02A74)

================================================================

<prolog><object-1>...<object-n><end-marker>

<prolog> = 02A74 (BIN5)

<object-i> = i-th object (ANY)

<end-marker> = 0312B (BIN5)

================================================================

Directory (02A96)

================================================================

<prolog><attachments><offset-1><end-dir>

<name-n><object-n><offset-n>...

<name-2><object-2><offset-2>

<name-1><object-1>

<prolog> = 02A96 (BIN5)

<attachments> = see comments (BIN3)

<end-dir> = 00000; end of directory marker (backwards scanned).

This is in the position that the <offset-n+1> field would

have occuppied (BIN5)

<offset-i> = offset (backward pointing) from the beginning of

the i'th offset field to the beginning of the i'th object's

name.  (BIN5)

<name-i> = the name of the i-th object (ASCIX)

<object-i> = the i'th object in the directory (ANY)

Comments:

The meaning of the <attachments> field is based on whether or

not the directory is the HOME directory.

HOME directory:  the <attachments> field contains the number of

libraries attached to the HOME directory, including the default,

'hidden' libraries: Library 2, standard functions; Library 1792,

flow control statments.

Subdirectory:  the <attachments> field contains the library

number of the library attached to this subdirectory or 7FF if

no library is attached.

================================================================

Algebraic (02AB8)

================================================================

<prolog> ... <end-marker>

<prolog> = 02AB8 (BIN5)

The "..." part is a sequence of objects and operations.

<end-marker> = 0312B (BIN5)

Example:

Create the expression 'A-2'.

02AB8		Algebraic

02E48 01 41	'A' (Global Name)

2A2DE		2 (address in ROM)

1AD09		-

0312B		End Marker

"8BA2084E201014ED2A209DA1B21303637" ASC\-> returns

'A-2'

================================================================

Unit (02ADA)

================================================================

<prolog><numeric-data><unit-1>...<unit-n><operation-1>...

<operation-n><end-marker>

<prolog> = 02ADA (BIN5)

<numeric-data> = the base data, if not Real Number, then will

be displayed as UNKNOWN (ANY)

<unit-i> = unit component (normally String or Real Number) (ANY)

<operation-j> = unit arithmetic indicator.  These indicators

point to null lists { }, but are interpreted by the OS as

simple unit operations (* / and so forth) (BIN5)

10B5E * (unit operator)

10B68 / (unit operator)

10B72 ^ (unit operator)

10B7C Character prefix (unit operator)

10B86 _ (unit operator)

<end-marker> = 0312B (BIN5)

================================================================

Tagged (02AFC)

================================================================

<prolog><name><object>

<prolog> = 02AFC (BIN5)

<name> = the tag (ASCIC)

<object> = the tagged object (ANY)

Example:

Create a the Real Number 1.23456789012 with a tag of "ABC"

02AFC 03 41 42 43 02933 0123456789012000

"CFA20301424343392000021098765432107141" ASC\-> returns

ABC: 1.23456789012

================================================================

Graphic (02B1E)

================================================================

<prolog><size><#rows><#columns><data>

<prolog> = 02B1E (BIN5)

<size> = number of nibbles remaining (BIN5)

<#rows> = number of pixel rows (BIN5)

<#columns> = number of pixel columns (BIN5)

<data> = graphic data, <size>-15 nibbles (BINx)

Each pixel in the Graphic is represented by a bit in <data>.

The pixels are grouped first by row and then by column.

The pixels for each row run for left to right and the bits for

those pixels run from least significant bit to most significant

bit.  Note that each row of pixels is padded to an even number

of nibbles.  For example:  a normal 131 X 64 Graphic requires

34 nibbles (136 bits) per row for a total of 2176 nibbles (not

counting the overhead of 20 nibbles).

X = <#columns> - 1

Y = <#rows> - 1

0                       X

+-------------------------+

0 |                         |

|                         |

|         Display         |

|                         |

Y |                         |

+-------------------------+

M = <#rows> - 1

N = 2*CEIL(<#columns>/8) - 1

N' = unpadded row size

??? = padding

N  N'                     0

+---+-->>--------------------+

|???|                        |  Row 0

+---+--<<--------------------+

|???|                        |  Row 1

+---+-->>--------------------+

:			:

+---+-->>--------------------+

|???|                        |  Row M

+---+--<<--------------------+

================================================================

Library (02B40)

================================================================

<prolog><size><library-name><library-number><hash-table-offset>

<message-table-offset><link-table-offset><config-offset>

<bulk-of-library><checksum>

<prolog> = 02B40 (BIN5)

<size> = number of nibbles remaining (BIN5)

<library-name> = Extended ASCII counted character string (ASCIX)

<library-number> = library number (BIN3)

<hash-table-offset> = offset to hash table (BIN5)

<message-table-offset> = offset to message table (BIN5)

<link-table-offset> = offset to link table (BIN5)

<config-offset> = offset to configuration code (BIN5)

<bulk-of-library> : <hash-table>, <message-table>, <link-table>,

<config-code> and the rest (in any order)

<checksum> = internal library checksum (BIN4)

<hash-table> : <ht-prolog><ht-size><offset-1>...<offset-16>

<nt-size><nt-entry-1>...<nt-entry-n><nt-offset-1>...

<nt-offset-n>

<ht-prolog> = 02A4E (Binary Integer) (BIN5)

<ht-size> = size of hash table without the prolog (BIN5)

<offset-i> = offset into the name table for the first command

with an i-character name (BIN5)

<nt-size> = size in nibbles of the name table (BIN5)

<nt-entry-j> : <command-name><xlib-number>

<command-name> = ASCII counted character string (ASCIC)

<xlib-number> = XLIB command number (BIN3)

<nt-offset-k> = offset to <nt-entry-j> for XLIB command k (BIN5)

This offset is backward pointing.

<message-table> = Array of String.  Each array element is a

message.

<link-table> : <lt-prolog><lt-size><lt-entry-1>...<lt-entry-n>

<lt-prolog> = 02A4E (BIN5)

<lt-size> = size of link table without the prolog (BIN5)

Library (cont)

<lt-entry-i> = offset to i-th command in this library (BIN5)

<config-code> = RPL subprogram that performs configuration

commands

Comments:

The hash table and link table are very large Binary Integers.

The message table is an Array of String.

The hash-table, message-table, link-table and config may be

referenced through a System Binary.  For example, the hash-table

offset could point to a System Binary which in turn points to

the real hash-table.

Although the HP 48SX does not contain any Library objects

itself, it clearly makes use of libraries.  It turns out that

the HP $*SX does contain part of the library header.  I call

these things "short libraries".  You can unthread short

libraries with the Voyager command EXAMINE LIBRARY.

<short-library> : <library-number><hash-table-offset>

<message-table-offset><link-table-offset><config-offset>

Example:

Unthread part of the stop watch library.  See the Voayager

Reference Manual for details on Voyager commands.

Voyager> load 1 sw-a

Voyager> ut 80000

80000: ; *** Library 768: SW-A ***

80000: 02B40 ! Library

80005: 01608 ! 5640 nibbles (next RPL at 8160D)

8000A: [...] ! Library name: "SW-A"

80016: 300   ! Library 300 (XLIB 768)

80019: 00014 ! Hash table at 8002D

8001E: 001F2 ! Message table at 80210

80023: 00107 ! Link table at 8012A

80028: 01543 ! Configuration code at 8156B

Voyager> exam hash 8002D 300

8002D: ; *** Hash table for library 300 (XLIB 768) ***

8002D: 02A4E ! (Binary Integer)

80032: 000F8 ! 248 nibbles (next at 8012A)

80037: 00000 !    1 none

8003C: 00000 !    2 none

80041: 00000 !    3 none

80046: 00046 !    4 at 8008C

8004B: 00000 !    5 none

80050: 00000 !    6 none

80055: 00085 !    7 at 800DA

8005A: 00093 !    8 at 800ED

8005F: 00000 !    9 none

80064: 00000 !   10 none

80069: 00000 !   11 none

8006E: 00000 !   12 none

Library (cont)

80073: 00000 !   13 none

80078: 00000 !   14 none

8007D: 00000 !   15 none

80082: 00000 !   16 none

80087: 0007B ! 123 nibbles (next at 80102)

8008C: [...] !   DOSW (XLIB 768 0)

80099: [...] !   PALL (XLIB 768 5)

800A6: [...] !   PSP+ (XLIB 768 4)

800B3: [...] !   VALL (XLIB 768 3)

800C0: [...] !   VSP+ (XLIB 768 1)

800CD: [...] !   VSP- (XLIB 768 2)

800DA: [...] !   RESETSW (XLIB 768 7)

800ED: [...] !   RCLSPLIT (XLIB 768 6)

80102: 00076 !   name for XLIB 768 0 at 8008C

80107: 00047 !   name for XLIB 768 1 at 800C0

8010C: 0003F !   name for XLIB 768 2 at 800CD

80111: 0005E !   name for XLIB 768 3 at 800B3

80116: 00070 !   name for XLIB 768 4 at 800A6

8011B: 00082 !   name for XLIB 768 5 at 80099

80120: 00033 !   name for XLIB 768 6 at 800ED

80125: 0004B !   name for XLIB 768 7 at 800DA

Voyager> exam link 8012A 300

8012A: ; *** Link table for library 300 (XLIB 768) ***

8012A: 02A4E ! (Binary Integer)

8012F: 000E1 ! 225 nibbles (next at 80210)

80134: 00167 !   XLIB 768 0 at 8029B

80139: 00183 !   XLIB 768 1 at 802BC

8013E: 001A4 !   XLIB 768 2 at 802E2

80143: 001C5 !   XLIB 768 3 at 80308

80148: 001E1 !   XLIB 768 4 at 80329

8014D: 00202 !   XLIB 768 5 at 8034F

80152: 0021E !   XLIB 768 6 at 80370

80157: 0023F !   XLIB 768 7 at 80396

8015C: 00254 !   XLIB 768 8 at 803B0

80161: 002DE !   XLIB 768 9 at 8043F

80166: 00303 !   XLIB 768 10 at 80469

8016B: 00328 !   XLIB 768 11 at 80493

80170: 0037D !   XLIB 768 12 at 804ED

80175: 0044F !   XLIB 768 13 at 805C4

8017A: 004E7 !   XLIB 768 14 at 80661

8017F: 00500 !   XLIB 768 15 at 8067F

80184: 00514 !   XLIB 768 16 at 80698

80189: 005BC !   XLIB 768 17 at 80745

8018E: 005E1 !   XLIB 768 18 at 8076F

80193: 00605 !   XLIB 768 19 at 80798

80198: 0069D !   XLIB 768 20 at 80835

8019D: 006CD !   XLIB 768 21 at 8086A

801A2: 0072A !   XLIB 768 22 at 808CC

801A7: 00753 !   XLIB 768 23 at 808FA

801AC: 00792 !   XLIB 768 24 at 8093E

801B1: 00819 !   XLIB 768 25 at 809CA

801B6: 00832 !   XLIB 768 26 at 809E8

801BB: 008B1 !   XLIB 768 27 at 80A6C

801C0: 008DA !   XLIB 768 28 at 80A9A

801C5: 00E8C !   XLIB 768 29 at 81051

Library (cont)

801CA: 00F55 !   XLIB 768 30 at 8111F

801CF: 00F95 !   XLIB 768 31 at 81164

801D4: 00FBC !   XLIB 768 32 at 81190

801D9: 00FE7 !   XLIB 768 33 at 811C0

801DE: 010AC !   XLIB 768 34 at 8128A

801E3: 0111D !   XLIB 768 35 at 81300

801E8: 0113A !   XLIB 768 36 at 81322

801ED: 01155 !   XLIB 768 37 at 81342

801F2: 0116A !   XLIB 768 38 at 8135C

801F7: 011EE !   XLIB 768 39 at 813E5

801FC: 0136F !   XLIB 768 40 at 8156B

80201: 01383 !   XLIB 768 41 at 81584

80206: 01396 !   XLIB 768 42 at 8159C

8020B: 013A9 !   XLIB 768 43 at 815B4

Voyager> exam message 80210 300

80210: ; *** Message table for library 300 (XLIB 768) ***

80210: 029E8 ! Array

80215: 0007F ! 127 nibbles

8021A: 02A2C ! ...of String

8021F: 00001 ! 1 dimensional

80224: 00003 ! Dim-1 = 3

. . . . . . ! 3 total elements

80229: [...] ! "Invalid SWDAT"

80248: [...] ! "Invalid Split#"

80269: [...] ! "Invalid Environment"

Voyager> ut 8156B

8156B: ; *** Configuaration code for library 300 (XLIB 768 40) ***

8156B: 02D9D ! Program

81570: [...] ! <300h>

8157A: 07709 ; Internal ATTACH to HOME directory (1:System Binary)

8157F: 0312B ! End Marker

Voyager> ut 8029B

8029B: ; *** DOSW (XLIB 768 0) ***

8029B: 02D9D ! Program

802A0: 18A1E ; Save current command, stack size, clear @706FD.S

802A5: [...] ! XLIB 768 8

802B0: 0312B ! End Marker

Voyager> exit

================================================================

Backup (02B62)

================================================================

<prolog><size><name><data><???>

<prolog> = 02B62 (BIN5)

<size> = number of nibbles remaining (BIN5)

<name> = the name of the backup (ASCIX)

<data> = the backed-up data (ANY)

<???> = a System Binary, possably a checksum.

================================================================

Library Data (02B88)

================================================================

<prolog><size><ident><data-1>...<data-n><end-marker>

<prolog> = 02B88 (BIN5)

<size> = number of nibbles remaining (BIN5)

<ident> = <library-number><object-number> (BIN5)

<library-number> = library number (BIN3)

<object-number> = object number (library specific) (BIN2)

<data-i> = the 'hidden' library data (ANY)

<end-marker> = 0312B (BIN5)

Example:

What's in 'MHpar' (MINEHUNT parameter) (EQ Library required)?

Download a copy of MHpar to your PC:

MINEHUNT STO

'MHpar' SEND

Revice MHpar and convert it to a Voyager MEM file:

C> kermit receive

C> bintomem mhpar.

Use Voyager to examine MHpar:

C> voyager

Voyager> load 1 mhpar

Voyager> ut 80000

80000: 02B88 ! Library Data

80005: 009C6 ! 2502 nibbles (next RPL at 809CB)

8000A: 0010C ! ident: 10C/00 (268/0)

8000F: 02A74 ! List

. . . . . . ! {

80014: 02B1E ! Graphic

80019: 0088F ! 2191 nibbles (next at 808A8)

8001E: 00040 ! 64 rows

80023: 00088 ! 136 columns

80028: [...] ! 00002D9C080445A3300003BB9306000000

8004A: [...] ! 8D106545141C6DA80000888AA80400CD00

8006C: [...] ! 8300EDDD04145DB31000098A9304000E00

8008E: [...] ! CF00A545141445B020000A8AA804008F10

800B0: [...] ! 20102D55180445AB100081BBA30E004020

800D2: [...] ! CF00000000000000000000000000008F10

800F4: [...] ! 0000000000000000000000000000000000

80116: [...] ! 00EFFFFFFFFFFFFFFFFFFFFFFFFFFF3000

80138: [...] ! 00EF180402018040201804020180402000

8015A: [...] ! 00EC180402018040201804020180402000

8017C: [...] ! 0068180402018040201804020180402000

Library Data (cont)

8019E: [...] ! 4468180402018040201804020180402110

801C0: [...] ! C6EC180402018040201804020180402B10

801E2: [...] ! 45EF180402018040201804020180402510

80204: [...] ! 44EFFFFFFFFFFFFFFFFFFFFFFFFFFF3110

80226: [...] ! 4420180402018040201804020180402110

80248: [...] ! 0020180402018040201804020180402000

8026A: [...] ! C720180402018040201804020180402F10

8028C: [...] ! 0120180402018040201804020180402400

802AE: [...] ! 0120180402018040201804020180402400

802D0: [...] ! 0120180402018040201804020180402400

802F2: [...] ! C7EFFFFFFFFFFFFFFFFFFFFFFFFFFF3F10

80314: [...] ! 0020180402018040201804020180402000

80336: [...] ! 4420180402018040201804020180402110

80358: [...] ! C420180402018040201804020180402310

8037A: [...] ! 4520180402018040201804020180402510

8039C: [...] ! 4620180402018040201804020180402910

803BE: [...] ! 4420180402018040201804020180402110

803E0: [...] ! 00EFFFFFFFFFFFFFFFFFFFFFFFFFFF3000

80402: [...] ! C720180402018040201804020180402F10

80424: [...] ! 4020180402018040201804020180402100

80446: [...] ! C320180402018040201804020180402F00

80468: [...] ! 4020180402018040201804020180402100

8048A: [...] ! C720180402018040201804020180402F10

804AC: [...] ! 0020180402018040201804020180402000

804CE: [...] ! 44EFFFFFFFFFFFFFFFFFFFFFFFFFFF3110

804F0: [...] ! 4420180402018040201804020180402110

80512: [...] ! C720180402018040201804020180402F10

80534: [...] ! 4420180402018040201804020180402110

80556: [...] ! 4420180402018040201804020180402110

80578: [...] ! 0020180402018040201804020180402000

8059A: [...] ! 4420180402018040201804020180402110

805BC: [...] ! 44EFFFFFFFFFFFFFFFFFFFFFFFFFFF3110

805DE: [...] ! 4420180402018040201804020180402110

80600: [...] ! 4420180402018040201804020180402110

80622: [...] ! 8320180402018040201804020180402E00

80644: [...] ! 0020180402018040201804020180402000

80666: [...] ! 4420180402018040201804020180402110

80688: [...] ! C420180402018040201804020180402310

806AA: [...] ! 45EFFFFFFFFFFFFFFFFFFFFFFFFFFF3510

806CC: [...] ! 4620180402018040201804020180402910

806EE: [...] ! 4420180402018040201804020180402110

80710: [...] ! 0020180402018040201804020180402000

80732: [...] ! C720180402018040201804020180402F10

80754: [...] ! 0120180402018040201804020180402400

80776: [...] ! 0120180402018040201804020180402400

80798: [...] ! 01EFFFFFFFFFFFFFFFFFFFFFFFFFFF3400

807BA: [...] ! 0120180402018040201804020180CA2400

807DC: [...] ! 0120180402018040201804020180453400

807FE: [...] ! 0020180402018040201804020180CA2000

80820: [...] ! 0020180402018040201804020180453000

80842: [...] ! 0020180402018040201804020180CA2000

80864: [...] ! C720180402018040201804020180453F10

80886: [...] ! EFEFFFFFFFFFFFFFFFFFFFFFFFFFFFBF30

. . . . . . ! End of Graphic

808A8: 03FF9 ; <1h>

808AD: 03FF9 ; <1h>

808B2: 03FF9 ; <1h>

Library Data (cont)

808B7: [...] ! "20000000001001000001000000000010

80901: [...] ! 00001101000110000000001000100001

80941: [...] ! 00000010001000000000000000001001

80981: [...] ! 00000010010000000001000000010000"

809C1: 0312B ! End Marker

. . . . . . ! }

809C6: 0312B ; End Marker

. . . . . . ! End of Library Data

Voyager> exit

================================================================

Program (02D9D)

================================================================

<prolog> ... <end-marker>

<prolog> = 02D9D (BIN5)

The "..." part is a sequence of objects.

<end-marker> = 0312B (BIN5)

Examples:

1) Write a strictly user-mode program to drop the stack level

two object.  This operation is called NIP by some people.

02D9D	Program

2361E	\<<

1FBBD	  SWAP

1FBD8	  DROP

23639	\>>

0312B	End Marker

"D9D29E1632DBBE18DDF193632B21306A65" ASC\-> returns

\<< SWAP DROP \>>

2) Write the same program using internal routines, with error

checking (standalone version, non-library).

02D9D	Program

18A8D	Verify DEPTH >= 2

60F9B	drop level two object

0312B	End Marker

"D9D20D8A81B9F06B2130D01D" ASC\-> returns

External External

================================================================

Code (02DCC)

================================================================

<prolog><size><machine-code>

<prolog> = 02DCC (BIN5)

<size> = number of nibbles remaining (BIN5)

<machine-code> = machine code (<size>-5 nibbles) (CODE)

================================================================

Global Name (02E48)

================================================================

<prolog><name>

<prolog> = 02E48 (BIN5)

<name> = ASCII counted character string (ASCIC)

Example:

Create a Global Name called 'GBL'.

02E48 03 47 42 4C

"84E20307424C4A205" ASC\-> returns 'GBL'

================================================================

Local Name (02E6D)

================================================================

<prolog><name>

<prolog> = 02E6D (BIN5)

<name> = ASCII counted character string (ASCIC)

Example:

Create a Local Name called 'LCL'.

02E6D 03 4C 43 4C

"D6E2030C434C4DCA1" ASC\-> returns 'LCL'

Executing EVAL at this point will generate an 'EVAL Error:

Undefined Local Name'.

================================================================

XLIB Name (02E92)

================================================================

<prolog><library-number><object-number>

<prolog> = 02E92 (BIN5)

<library-number> = library number (BIN3)

<object-number> = object number with library (BIN3)

Example:

The XLIB name for the EQ Library's MINEHUNT is XLIB 268 0

02E92 10C 000

If you have the EQ Library:

"29E20C01000FEA7" ASC\-> returns MINEHUNT

If you don't:

"29E20C01000FEA7" ASC\-> returns XLIB 268 0