12     RFRV20     08/29/89 12:45:45     WACCVM2 MAIL

_______________(Message From PROFS on 08/29/89 at 12:45:38)___________

MC68HC11

Floating Point Package

Written By

Gordon Doughman

1986 Gordon Doughman

1.0 Introduction:

The MC68HC11 is a very powerful and capable single chip microcomputer.  Its concise instruction set

combined with 6 powerful addressing modes, true bit manipulation, 16-bit arithmetic operations and a second 16-bit

index register make it ideal for control applications requiring  both high speed I/O and high speed calculations.

While most applications can be implemented by using the 16-bit integer precision of the MC68HC11,

certain applications or algorithms may be difficult or impossible to implement without floating point math.  The goal in

writing the MC68HC11 floating point package was to provide a fast, flexible way to do floating point math for just such

applications.

The HC11 floating point package (HC11FP) implements more than just the four basic math functions (add,

subtract, multiply, and divide) it also provides routines to convert from ASCII to floating point and from floating point

to ASCII.  For those applications that require it, the three basic trig functions SINe, COSine, and TANgent are

provided along with some trig utility functions for converting to and from both radians and degrees.  The square root

function is also included.

For those applications that can benefit by using both integer and floating point operations, there are

routines to convert to and from integer and floating point format.

The entire floating point package requires just a little over 2k bytes of memory and only requires ten

bytes of page zero RAM in addition to stack RAM.  All temporary variables needed by the floating point routines,

reside on the stack.  This feature makes the routines completely reentrant as long as the ten bytes of page zero RAM

are saved before using any of the routines.  This will allow both interrupt routines and main line programs to use the

floating point package with out interfering with one another.

2.0 Floating Point Format:

2.1 Floating Point Accumulator Format:

The ten bytes of page zero RAM are used for two software floating point accumulators FPACC1 and

FPACC2.  Each 5 byte accumulator consists of a one byte exponent, a three byte mantissa, and one byte that is used

to indicate the mantissa sign.

The exponent byte is used to indicate the position of the binary point  and is biased by decimal 128 (80)

to make floating point comparisons easier. This one byte exponent gives a dynamic range of about 1  1038.

The mantissa consists of three bytes (24 bits) and is used to hold both the integer and fractional portion

of the floating point number.  The mantissa is always assumed to be "normalized" (i.e. most significant bit of the most

significant byte a 1).  A twenty-four bit mantissa will provide slightly more than seven decimal digits of precision.

A separate byte is used to indicate the sign of the mantissa rather than keeping it in two's complement

form so that unsigned arithmetic operations may be used when manipulating the mantissa.  A positive mantissa is

indicated by this byte being equal to zero (00).  A negative mantissa is indicated by this byte being equal to -1 (FF).

FPACC1  82 C90FDB 00    +3.1415927

FPACC2  82 C90FDB FF    -3.1415927

2.2 "Memory Format":

The way that floating point numbers are stored in memory or the "memory format" of a floating point

number is slightly different than its floating point accumulator format.  In order to save memory, floating point numbers

are stored in memory in a format called "hidden bit normalized form".

In this format, the number is stored into four consecutive bytes with the exponent residing at the lowest

address.  The mantissa is stored in the next three consecutive bytes with the most significant byte stored in the

lowest address.  Since the most significant bit of the mantissa in a normalized floating point number is always "1", this

bit can be used to store the sign of the mantissa.  This results in positive numbers having the most significant bit of

the mantissa cleared (0) and negative numbers having their most significant bit set (1).  An example follows:

82 490FDB       +3.1415927

82 C90FDB       -3.1415927

There are four routines that can be used to save and load the floating point accumulators and at the same

time convert between the floating point accumulator and memory format.  These routines are discussed in detail in

another section.

3.0 Errors:

There are 7 error conditions that may be returned by the HC11 floating point package.  When an error

occurs, the condition is indicated to the calling program by setting the carry bit in the condition code register and

returning an error code in the A-accumulator.  The error codes and their meanings are explained below.

Error # Meaning

1       Format error in ASCII to floating point conversion.

2       Floating point overflow.

3       Floating point underflow.

4       Division by zero (0).

5       Floating point number too large or small to convert

to integer.

6       Square root of a negative number.

7       TAN of /2 (90)

Note: None of the routines check for valid floating point numbers in either FPACC1 or FPACC2.  Having illegal

floating point values in the floating point accumulators will produce unpredictable results.

4.0 Floating Point Routines:

The following sections provides a description of each routine in the floating point package.  The

information provided includes the subroutine name, operation performed, subroutine size, stack space required,

other subroutines that are called, input, output, and possible error conditions.

The Stack Space required by the subroutine includes not only that required for the particular routines

local variables, but also stack space that is used by any other subroutines that are called including return addresses.

Note that the trig functions require a good deal of stack space.

Since some applications may not require all the routines provided in the floating point package, the

description of each routine includes a section giving the names of other subroutines that it calls.  This makes it easy

to determine exactly which subroutines are required for a particular function.

4.1 ASCII-to-Floating Point Conversion:

Subroutine Name:        ASCFLT

Operation:      ASCII (X)  FPACC1

Size:   352 Bytes (includes NUMERIC subroutine)

Stack Space:    14 Bytes

Calls:  NUMERIC, FPNORM, FLTMUL, PSHFPAC2, PULFPAC2

Input:  X-register points to ASCII string to convert.

Output: FPACC1 contains the floating point number.

Error conditions:       Floating point format error may be returned.

Notes:  This routine converts an ASCII floating point number to the format required by all of

the floating point routines.  Conversion stops either when a non-decimal character is

encountered before the exponent or after one or two exponent digits have been

converted.  The input format is very flexible.  Some examples are shown below.

20.095

0.125

7.2984E+10

167.824E5

005.9357E-7

500

4.2 Floating Point Multiply:

Subroutine Name:        FLTMUL

Operation:      FPACC1  FPACC2  FPACC1

Size:   169 Bytes

Stack Space:    10 Bytes

Calls:  PSHFPAC2, PULFPAC2, CHCK0

Input:  FPACC1 and FPACC2 contain the numbers to be multiplied.

Output: FPACC1 contains the product of the two floating point accumulators.  FPACC2

remains unchanged.

Error conditions:       Overflow, Underflow.

4.3 Floating Point Add:

Subroutine Name:        FLTADD

Operation:      FPACC1 + FPACC2  FPACC1

Size:   194 Bytes

Stack Space:    6 Bytes

Calls:  PSHFPAC2, PULFPAC2, CHCK0

Input:  FPACC1 and FPACC2 contain the numbers to be added.

Output: FPACC1 contains the sum of the two numbers.  FPACC2 remains unchanged.

Error conditions:       Overflow, Underflow.

Notes:  The floating point add routine performs full signed addition.  Both floating point

accumulators may have mantissas  with the same or different sign.

4.4 Floating Point Subtract:

Subroutine Name:        FLTSUB

Operation:      FPACC1 - FPACC2  FPACC1

Size:   12 Bytes

Stack Space:    8 Bytes

Calls:  FLTADD

Input:  FPACC1 and FPACC2 contain the numbers to be subtracted.

Output: FPACC1 contains the difference of the two numbers (FPACC1-FPACC2).  FPACC2

remains unchanged.

Error conditions:       Overflow, Underflow.

Notes:  Since FLTADD performs full signed addition, the floating point subtract routine

inverts the sign byte of FPACC2, calls FLTADD and then changes the sign of

FPACC2 back to what it was originally.

4.5 Floating Point Divide:

Subroutine Name:        FLTDIV

Operation:      FPACC1  FPACC2  FPACC1

Size:   209 Bytes

Stack Space:    11 Bytes

Calls:  PSHFPAC2, PULFPAC2

Input:  FPACC1 and FPACC2 contain the divisor and dividend respectively.

Output: FPACC1 contains the quotient. FPACC2 remains unchanged.

Error Conditions:       Divide by zero, Overflow, Underflow

4.6 Floating Point to ASCII Conversion:

Subroutine Name:        FLTASC

Operation:      FPACC1  (X)

Size:   370 Bytes

Stack Space:    28 Bytes

Calls:  FLTMUL, FLTCMP, PSHFPAC2, PULFPAC2

Input:  FPACC1 contains the number to be converted to an ASCII string. The index register

X points to a 14 byte string buffer.

Output: The buffer pointed to by the X index register contains an ASCII string that represents

the number in FPACC1.  The string is terminated with a zero (0) byte and the X

register points to the start of the string.

Error Conditions:       None.

4.7 Floating Point Compare:

Subroutine Name:        FLTCMP

Operation:      FPACC1 - FPACC2

Size:   42 Bytes

Stack Space:    None

Calls:  None

Input:  FPACC1 and FPACC2 contain the numbers to be compared.

Output: Condition codes are properly set so that all branch instructions may be used to alter

program flow.  FPACC1 and FPACC2 remain unchanged.

Error Conditions:       None.

4.8 Unsigned Integer to Floating Point:

Subroutine Name:        UINT2FLT

Operation:      (16-bit unsigned integer)  FPACC1

Size:   18 Bytes

Stack Space:    6 Bytes

Calls:  FPNORM, CHCK0

Input:  The lower 16-bits of the FPACC1 mantissa contain an unsigned 16-bit integer.

Output: FPACC1 contains the floating point representation of the 16-bit unsigned integer.

Error Conditions:       None.

4.9 Signed Integer to Floating Point:

Subroutine Name:        SINT2FLT

Operation:      (16-bit signed integer)  FPACC1

Size:   24 Bytes

Stack Space:    7 Bytes

Calls:  UINT2FLT

Input:  The lower 16-bits of the FPACC1 mantissa contain a signed 16-bit integer.

Output: FPACC1 contains the floating point representation of the 16-bit signed integer.

Error Conditions:       None.

4.10 Floating Point to Integer:

Subroutine Name:        FLT2INT

Operation:      FPACC1  (16-bit signed or unsigned integer)

Size:   74 Bytes

Stack Space:    2 Bytes

Calls:  CHCK0

Input:  FPACC1 may contain a floating point number in the range 65535  FPACC1  -32767.

Output: The lower 16-bits of the FPACC1 mantissa will contain a 16-bit signed or unsigned

number.

Error Conditions:       None.

Notes:  If the floating point number  in FPACC1 is positive, it will be converted to an unsigned

integer.  If the number is negative it will be converted to a signed twos complement

integer. This type of conversion will allow 16-bit addresses to be represented as

positive numbers in floating point format.  Any fractional part of the floating point

number is discarded.

4.11 Transfer FPACC1 to FPACC2:

Subroutine Name:        TFR1TO2

Operation:      FPACC1  FPACC2

Size:   13 Bytes

Stack Space:    0 Bytes

Calls:  None

Input:  FPACC1 contains a floating point number.

Output: FPACC2 contains the same number as FPACC1.

Error Conditions:       None.

5.0 Floating Point Functions:

The following section describes the supplied floating point functions, returned results, and possible error

conditions.  Note that even though the Taylor series which is used to calculate the trig functions requires that the

input angle be expressed in radians, less precision is lost through angle reduction if the angle being reduced is

expressed in degrees. Once the angle is reduced, the DEG2RAD subroutine is called to convert the angle to radians.

To reduce the number of factors in the Taylor expansion series all angles are reduced to fall between 0

and 45 by the ANGRED subroutine. This subroutine returns the reduced angle in FPACC1 along with the quad number

that the original angle was in and a flag that tells the calling routine whether it actually needs to calculate the sine or

the cosine of the reduced angle to obtain the proper answer.

5.1 Square Root:

Subroutine Name:        FLTSQR

Operation:      FPACC1  FPACC1

Size:   104 Bytes

Stack Space:    21 Bytes

Calls:  TFR1TO2, FLTDIV, FLTADD, PSHFPAC2, PULFPAC2

Input:  FPACC1 contains a valid floating point number.

Output: FPACC1 contains the square root of the original number.  FPACC2 is unchanged.

Error Conditions:       NSQRTERR is returned if the number in FPACC1 is negative and FPACC1 remains

unchanged.

5.2 Sine:

Subroutine Name:        FLTSIN

Operation:      SIN(FPACC1)  FPACC1

Size:   380 Bytes ( Includes SINCOS subroutine )

Stack Space:    50 Bytes

Calls:  ANGRED, SINCOS, DEG2RAD, PSHFPAC2, PULFPAC2

Input:  FPACC1 contains an angle in radians in the range -2  FPACC1  +2.

Output: FPACC1 contains the sine of FPACC1 and FPACC2 remains unchanged.

Error Conditions:       None.

Notes:  The Taylor Expansion Series is used to calculate the sine of the angle between 0 and

45 (4).  The subroutine ANGRED is called to reduce the input angle to within this

range.  Spot checks show a maximum error of +1.510-7 throughout the input range.

5.3 Cosine:

Subroutine Name:        FLTCOS

Operation:      COS(FPACC1)  FPACC1

Size:   384 Bytes ( Includes SINCOS subroutine )

Stack Space:    50 Bytes

Calls:  ANGRED, FLTSIN, DEG2RAD, PSHFPAC2

Input:  FPACC1 contains an angle in radians in the range -2  FPACC1  +2.

Output: FPACC1 contains the cosine of FPACC1 and FPACC2 remains unchanged.

Error Conditions:       None.

Notes:  The Taylor Expansion Series is used to calculate the cosine of the angle between 0

and 45 (4).  The subroutine ANGRED is called to reduce the input angle to within

this range.  Spot checks show a maximum error of +1.510-7 throughout the input

range.

5.4 Tangent:

Subroutine Name:        FLTTAN

Operation:      TAN(FPACC1)  FPACC1

Size:   35 Bytes ( Also requires FLTSIN and FLTCOS )

Stack Space:    56 Bytes

Calls:  TFR1TO2, EXG1AND2, FLTSIN, FLTCOS, FLTDIV, PSHFPAC2, PULFPAC2

Input:  FPACC1 contains an angle in radians in the range -2  FPACC1  +2.

Output: FPACC1 contains the tangent of the input angle and FPACC2 remains unchanged.

Error Conditions:       Returns largest legal number if tangent of 2 is attempted.

Notes:  The tangent of the input angle is calculated by first obtaining the sine and cosine of

the input angle and  then using the following formula: TAN=SINCOS.  At 89.9 the

tangent function is only accurate to 5 decimal digits.  For angles greater than 89.9

accuracy decreases rapidly.

5.5 Degrees to Radians Conversion:

Subroutine Name:        DEG2RAD

Operation:      FPACC1    180  FPACC1

Size:   15 Bytes

Stack Space:    16 Bytes

Calls:  GETFPAC2,FLTMUL

Input:  Any valid floating point number representing an angle in degrees.

Output: Input angles equivalent in radians.

Error Conditions:       None.

5.6 Radians to Degrees Conversion:

Subroutine Name:        RAD2DEG

Operation:      FPACC1  180    FPACC1

Size:   8 Bytes ( Also requires DEG2RAD subroutine )

Stack Space:    16 Bytes

Calls:  DEG2RAD

Input:  Any valid floating point number representing an angle in radians.

Output: Input angles equivalent in degrees.

Error Conditions:       Overflow, Underflow.

5.7 PI:

Subroutine Name:        GETPI

Operation:        FPACC1

Size:   6 Bytes

Stack space:    None

Input:  None

Output: The value of  is returned in FPACC1.

Error Conditions:       None

Notes:  This routine should be used to obtain the value of  if it is required in calculations

since it is accurate to the full 24 bits of the mantissa.

6.0 Format Conversion Routines:

As discussed in section 2.1 and 2.2 the format for floating point numbers as they appear in the floating

point accumulators is different than the way numbers are stored in memory.  This was done primarily to save memory

when a large number of floating point variables are used in a program.  Four routines are provided to convert to and

from the different formats while at the same time moving a number into or out of the floating point accumulators.  By

always using these routines to move numbers into and out of the floating point accumulators it would be extremely

easy to adapt this floating point package to work with any other floating point format.

One example might be to interface this package with code produced by Motorola's 68HC11 'C' compiler.

The Motorola 'C' compiler generates code for single precision floating point numbers whose internal format is that

defined by the IEEE Standard for Binary Floating Point Arithmetic.  By rewriting the four routines described below the

IEEE format could be easily converted to the format required by this floating point package.

6.1 Get FPACC(x):

Subroutine Name:        GETFPAC1 & GETFPAC2

Operation:      (X)  FPACC1; (X)  FPACC2

Size:   22 Bytes each.

Stack Space:    None

Input:  The X index register points to the 'memory formated' number to be moved into the

floating point accumulator.

Output: The number pointed to by X is in the specifyed floating point accumulator.

Error Conditions:       None

6.1 Put FPACC(x):

Subroutine Name:        PUTFPAC1 & PUTFPAC2

Operation:      FPACC1  (X); FPACC2  (X)

Size:   22 Bytes each.

Stack Space:    None

Input:  The X index register points to four consecutive memory locations where the number

will be stored.

Output: The floating point accumulator is moved into consecutive memory locations pointed

to by the X index register.

Error Conditions:       None